//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// Arduino DCC Solenoid Switch Decoder.

// Author: Ruud Boer - January 2015

// This sketch turns an Arduino into a DCC decoder to drive max 8 dual coil solenoid switches.

// The DCC signal is optically separated and fed to pin 2 (=Interrupt 0). Schematics: www.mynabay.com

// Many thanks to www.mynabay.com for publishing their DCC monitor and -decoder code.

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// IMPORTANT: GOTO lines 17 and 40 to configure some data!

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#include <DCC_Decoder.h>

#define kDCC_INTERRUPT 0

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// FILL IN

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

const byte maxaccessories=1; //The number of switches you want to control with this Arduino

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

typedef struct

{

int               address;         // Address to respond to

byte              output;          // State of accessory: 1=on, 0=off (for internal use only)

int               outputPin;       // Arduino output pin

int               outputPin2;      // Arduino output pin2, used for solenoid junctions

byte              highlow;         // State of outputpin: 1=HIGH, 0=LOW

byte              highlow2;        // State of outputpin2: 1=HIGH, 0=LOW

boolean           finished;        // Memory location that says the oneshot is finished

boolean           finished2;       // Memory location that says the second oneshot (for solenoid) is finished

int               durationMilli;   // ms flash time

unsigned long     onMilli;         // for internal use

unsigned long     offMilli;        // for internal use

}

DCCAccessoryAddress;

DCCAccessoryAddress accessory[maxaccessories];

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// Initialization: COPY - PASTE the structure as many times as you have switches and fill in the values you want.

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

void ConfigureDecoderFunctions() // The amount of accessories must be same as in line 26 above!

{

accessory[0].address = 1;

accessory[0].durationMilli = 500;

accessory[0].outputPin = 1;

accessory[0].outputPin2 = 3;

accessory[0].highlow = 0; // Do not change this value

accessory[0].highlow2 = 0; // Do not change this value

accessory[0].finished = false; // Do not change this value

accessory[0].finished2 = true; // Do not change this value

accessory[0].output = 0; // Do not change this value

// Setup output pins

for(int i=0; i<maxaccessories; i++)

{

if( accessory[i].outputPin )

{

pinMode( accessory[i].outputPin, OUTPUT );

digitalWrite( accessory[i].outputPin, LOW);

}

if( accessory[i].outputPin2 )

{

pinMode( accessory[i].outputPin2, OUTPUT );

digitalWrite( accessory[i].outputPin2, LOW);

}

}

} // END ConfigureDecoderFunctions

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// DCC packet handler

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

void BasicAccDecoderPacket_Handler(int address, boolean activate, byte data)

{

// Convert NMRA packet address format to human address

address -= 1;

address *= 4;

address += 1;

address += (data & 0x06) >> 1;

boolean enable = (data & 0x01) ? 1 : 0;

for(int i=0; i<maxaccessories; i++)

{

if( address == accessory[i].address )

{

if( enable ) accessory[i].output = 1;

else accessory[i].output = 0;

}

}

} // END BasicAccDecoderPacket_Handler

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// Setup (run once)

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

void setup()

{

DCC.SetBasicAccessoryDecoderPacketHandler(BasicAccDecoderPacket_Handler, true);

ConfigureDecoderFunctions();

DCC.SetupDecoder( 0x00, 0x00, kDCC_INTERRUPT );

pinMode(2,INPUT_PULLUP); //Interrupt 0 with internal pull up resistor (can get rid of external 10k)

pinMode(13,OUTPUT);

digitalWrite(13,LOW); //turn off Arduino led at startup

for (int n=0; n<maxaccessories; n++) accessory[n].output = 0; //all servo's to min angle and functions to 0

} //END setup

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// Main loop (run continuous)

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

void loop()

{

static int addr = 0;

DCC.loop(); // Loop DCC library

if( ++addr >= maxaccessories ) addr = 0; // Bump to next address to test

if (accessory[addr].output == 1)

{

if (!accessory[addr].highlow && !accessory[addr].finished)

{

accessory[addr].highlow = 1;

accessory[addr].offMilli = millis() + accessory[addr].durationMilli;

}

if (accessory[addr].highlow && millis() > accessory[addr].offMilli)

{

accessory[addr].highlow = 0;

accessory[addr].finished = true;

}

accessory[addr].finished2 = false;

}

else // output==0

{

accessory[addr].highlow=0;

accessory[addr].finished = false;

if (!accessory[addr].highlow2 && !accessory[addr].finished2)

{

accessory[addr].highlow2 = 1;

accessory[addr].offMilli = millis() + accessory[addr].durationMilli;

}

if (accessory[addr].highlow2 && millis() > accessory[addr].offMilli)

{

accessory[addr].highlow2 = 0;

accessory[addr].finished2 = true;

}

}

if (accessory[addr].highlow) digitalWrite( accessory[addr].outputPin, HIGH);

else digitalWrite( accessory[addr].outputPin, LOW);

if (accessory[addr].highlow2) digitalWrite( accessory[addr].outputPin2, HIGH);

else digitalWrite( accessory[addr].outputPin2, LOW);

} //END loop

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////